# Video Ground Loop Isolator GB001

Features

- BNC male to BNC female with mini cable 15 CM on both sides

- Passive, no power required

- Prevents video signal distortion caused by video ground loops

- Eliminates picture tearing, cross talk and rolling

- Built in TVS (Transcient Voltage Suppressors) for surge protection

The GB001 is a video ground loop isolator for coaxial cable and CCTV camera applications. The ground loop isolator has a BNC male to BNC female in/output. This video product is passive which means it does not require power. The idea behind is that it prevents video signal distortion caused by video ground loops. Video ground loop isolators are passive devices which reduce ground loop interference with CCTV signals and can easily install in a new or added to existing CCTV camera systems.  These devices are useful where a video signal is transmitted via coaxial cable between points with different ground potentials.  Differences in ground potentials are typically caused by unbalanced power line loads. Not weatherproof.

| Item: | Video Balun |
| --- | --- |
| Frequency response: | DC to 8MHz |
| Attenuation: | 0.5dB typ |
| CMRR: | 60dB typ |
| Video format: | NTSC, PAL and SECAM |
| Connector: | Male BNC |
| Impedance: | BNC: 75 ohms |
|  | Terminal: 100 ohms |
| Network Wiring: | One Unshielded Twisted-Pair 24-16 AWG (0.5 ~ 1.31mm) |
| Category Type: | 5 or better |
| Impedance: | 100 ± 20 ohms |
| DC Loop Resistance (24 AWG) : | 52 ohms per 1000 ft (18 ohms per 100m) |
| Differential Capacitance: | 19 pF/ft max ( 62pF/m max) |
| Working temperature: | -20°C ~ 75°C |
| Storage temperature: | -30°C ~ 85°C |
| Humidity: | 0% ~ 95% RH |
| Transient Immunity: | per ANSI / IEEE 587 C62.41 |
| Package: | 100 pairs/carton box |
| Warranty: | One Year |
| Features: | 1.Compact size and easy installation |
|  | 2.Surge/Lighting Protection |
|  | 3.Interference Suppression/ Noise Filtering |
|  | 4.Full-motion CCTV video at distances up to 330m when used with any passive transceiver |
|  | 5.Up to 1500m with any active UTP video receiver |
|  | 6.Male BNC allows connection directly to the camera or DVR |